System-2 Group Assignment

Instruction Byte Diagram:

| OPCODE | MOD R/M | SIB | DISPLACEMENT | IMMEDIATE |
| --- | --- | --- | --- | --- |

-- OPCODE:  A primary opcode can be 1, 2, or 3 bytes in length.

-- MOD R/M: Takes total 1 byte.

The Mod r/m byte is:

| MOD | REG/ OPCODE | R/M |
| --- | --- | --- |
| 2 bits | 3 bits | 3 bits |

-- SIB: Takes total 1 byte.

The SIB byte will be:

| Scale | Index | Base |
| --- | --- | --- |
| 2 bits | 3 bits | 3 bits |

-- Displacement: If a displacement is required, it can be 1, 2, or 4 bytes.

-- Immediate: An immediate operand can be 1, 2 or 4 bytes.

Encoding table:

32 bit Addressing forms with mod r/m byte

| REG | Register |
| --- | --- |
| 000 | EAX |
| 001 | ECX |
| 010 | EDX |
| 011 | EBX |
| 100 | ESP |
| 101 | EBP |
| 110 | ESI |
| 111 | EDI |

The Mod r/m byte is:

| MOD | REG/ OPCODE | R/M |
| --- | --- | --- |
| 2 bits | 3 bits | 3 bits |

Mod  = 00, [ Memory no displacement ]

| Effective Address | R/M | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [EAX] | 000 | 00 | 08 | 10 | 18 | 20 | 28 | 30 | 38 |
| [ECX] | 001 | 01 | 09 | 11 | 19 | 21 | 29 | 31 | 39 |
| [EDX] | 010 | 02 | 0A | 12 | 1A | 22 | 2A | 32 | 3A |
| [EBX] | 011 | 03 | 0B | 13 | 1B | 23 | 2B | 33 | 3B |
| [--][--] | 100 | 04 | 0C | 14 | 1C | 24 | 2C | 34 | 3C |
| disp32 | 101 | 05 | 0D | 15 | 1D | 25 | 2D | 35 | 3D |
| [ESI] | 110 | 06 | 0E | 16 | 1E | 26 | 2E | 36 | 3E |
| [EDI] | 111 | 07 | 0F | 17 | 1F | 27 | 2F | 37 | 3F |

Mod  = 01, [ Memory + 8 bit displacement ]

| Effective Address | R/M | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [EAX]+disp8 | 000 | 40 | 48 | 50 | 58 | 60 | 68 | 70 | 78 |
| [ECX+disp8 | 001 | 41 | 49 | 51 | 59 | 61 | 69 | 71 | 79 |
| [EDX]+disp8 | 010 | 42 | 4A | 52 | 5A | 62 | 6A | 72 | 7A |
| [EBX]+disp8 | 011 | 43 | 4B | 53 | 5B | 63 | 6B | 73 | 7B |
| [--][--]+disp8 | 100 | 44 | 4C | 54 | 5C | 64 | 6C | 74 | 7C |
| [EBP]+disp8 | 101 | 45 | 4D | 55 | 5D | 65 | 6D | 75 | 7D |
| [ESI]+disp8 | 110 | 46 | 4E | 56 | 5E | 66 | 6E | 76 | 7E |
| [EDI]+disp8 | 111 | 47 | 4F | 57 | 5F | 67 | 6F | 77 | 7F |

Mod  = 10, [ Memory + 32 bit displacement ]

| Effective Address | R/M | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [EAX]+disp32 | 000 | 80 | 88 | 90 | 98 | A0 | A8 | B0 | B8 |
| [ECX]+disp32 | 001 | 81 | 89 | 91 | 99 | A1 | A9 | B1 | B9 |
| [EDX]+disp32 | 010 | 82 | 8A | 92 | 9A | A2 | AA | B2 | BA |
| [EBX]+disp32 | 011 | 83 | 8B | 93 | 9B | A3 | AB | B3 | BB |
| [--][--]+disp32 | 100 | 84 | 8C | 94 | 9C | A4 | AC | B4 | BC |
| [EBP]+disp32 | 101 | 85 | 8D | 95 | 9D | A5 | AD | B5 | BD |
| [ESI]+disp32 | 110 | 86 | 8E | 96 | 9E | A6 | AE | B6 | BE |
| [EDI]+disp32 | 111 | 87 | 8F | 97 | 9F | A7 | AF | B7 | BF |

Mod  = 11, [ Memory, redister-direct]

| Effective Address | R/M | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EAX | 000 | C0 | C8 | D0 | D8 | E0 | E8 | F0 | F8 |
| ECX | 001 | C1 | C9 | D1 | D9 | E1 | E9 | F1 | F9 |
| EDX | 010 | C2 | CA | D2 | DA | E2 | EA | F2 | FA |
| EBX | 011 | C3 | CB | D3 | DB | E3 | EB | F3 | FB |
| ESP | 100 | C4 | CC | D4 | DC | E4 | EC | F4 | FC |
| EBP | 101 | C5 | CD | D5 | DD | E5 | ED | F5 | FD |
| ESI | 110 | C6 | CE | D6 | DE | E6 | EE | F6 | FE |
| EDI | 111 | C7 | CF | D7 | DF | E7 | EF | F7 | FF |

32 bit Addressing forms with SIB byte.

The SIB byte will be:

| Scale | Index | Base |
| --- | --- | --- |
| 2 bits | 3 bits | 3 bits |

/

// for now tried to keep it simple

| Scale | [ Index Displacement ] |
| --- | --- |
| 00 | x1 |
| 01 | x2 |
| 10 | x4 |
| 11 | x8 |

SIB Index table:

| Bits | Register |
| --- | --- |
| 000 | EAX |
| 001 | ECX |
| 010 | EDX |
| 011 | EBX |
| 100 | No index |
| 101 | EBP |
| 110 | ESI |
| 111 | EDI |

SIB Base table:

| Bits | Register |
| --- | --- |
| 000 | EAX |
| 001 | ECX |
| 010 | EDX |
| 011 | EBX |
| 100 | ESP |
| 101 | *EBP |
| 110 | ESI |
| 111 | EDI |

OPCODE table:

| Instruction | Operand Form | Opcode |
| --- | --- | --- |
| MOV | r/m32, imm32 | C7 /0 id |
| MOV | r/m32, r32 | 89 /r |
| ADD | r/m32, imm8 | 83 /0 ib |
| ADD | r/m32, imm32 | 81 /0 id |
| ADD | r/m32, r32 | 01 /r |
| SUB | r/m32, imm8 | 83 /5 ib |
| SUB | r/m32, imm32 | 81 /5 id |
| SUB | r/m32, r32 | 29 /r |
| DIV | r/m32 | F7 /6 |
| INC | r/m32 | FF /0 |
| DEC | r/m32 | FF /1 |
| PUSH | r32 | 50+rd |
| PUSH | r/m32 | FF /6 |
| POP | r32 | 58+rd |
| POP | r/m32 | 8F /0 |
| RET | - | C3 |

Group Members

Navnath More:  Mod R/M generation.

Navnath will handle the instruction-format and Mod R/M part of the assembler.

This includes identifying the instruction components such as Opcode, Mod R/M,

SIB, Displacement, and Immediate, and implementing the logic for constructing

the Mod R/M byte using the MOD, REG/OPCODE, and R/M fields.

Sachin Kharat:   SIB generation.

Sachin will handle SIB-based addressing. His work will include generating the SIB byte 		  using the Scale, Index, and Base fields and implementing the lookup rules documented 			  in the SIB tables.

Mayuri Khatal:   Opcode Selection.

Mayuri will handle opcode selection and instruction encoding using the opcode table 			   in the document. Her work will include identifying the instruction and operand form 			   and selecting the corresponding encoding, such as 89 /r, C7 /0 id, 83 /0 ib, 81 /5 id, 			   F7 /6, FF /0, 50+rd, 58+rd, and C3. She will also handle the interpretation of /r, /0–/7, 			   ib, and id according to the opcode entries listed in the document.

Shreyash Sawant:  Combine all bytes.

Shreyash will integrate the Opcode, ModR/M, and SIB components into the final 			     encoding process. His work will include combining the required Opcode, ModR/M 			     byte, SIB byte, Displacement, and Immediate fields to produce the final machine-			     code byte sequence. He will also handle the little-endian ordering of multi-byte 			     Displacement and Immediate values and test the complete assembler using the 			     instruction forms listed in the document.

Github link:	https://github.com/Navnath28/System-II

Member github link:

Navnath More: https://github.com/Navnath28

Sachin Kharat: https://github.com/kharatsachin46

Mayuri Khatal: https://github.com/mayuri5112

Shreyash Sawant: https://github.com/shreyash-hash